RTI VIP-UHD-CTRL Firmware Version 2.03

LIST OF CHANGES IN THIS RELEASE (RELEASE v2.03)

-------------------------------------------------------

- Ability to modify HDCP Mode.

  - Bypass: Follows the source encryption

    - Eg. If source is unencrypted output will also be unencrypted.

  - Force 2.2: Forces output to always encrypt with HDCP 2.2. Regardless of input encryption.

  - Force 1.4: Forces output to always encrypt with HDCP 1.4. Regardless of input encryption.

2. Ability to change polling frequency for large systems (75+ TX or RX) to improve stability.

Note: VIP-UHD-TX and VIP-UHD-RX MUST be on version 2.3.

INSTALLATION REQUIREMENTS

-------------------------

Updating the VIP-UHD-CTRL requires:

1. The VIP-UHD-CTRL to be connected to the same Ethernet network (LAN) as the PC running APEX.

NOTE: Hardwire connection is recommended.

FIRMWARE UPGRADE PROCEDURE

--------------------------

1. Download the VIP-UHD-CTRL firmware file (.bin) from the RTI website to your computer.

2. Open APEX file with VIP-UHD-CTRL that needs to be updated.

3. Click on the driver dropdown and then click on the “RTI VIP-UHD-CTRL” driver

4. In the APEX workspace, click on “Launch Configuration” to launch the VIP-UHD-CTRL GUI.

5. Log into VIP-UHD-CTRL with Username and password.

6. In the top bar, click “Update Firmware”.

7. In the top bar of the Update Firmware page, click on “Upload VIP-UHD-CTRL Firmware”.

8. Select the [VIP-UHD-CTRL].bin file downloaded to your computer for the unit. The file will automatically upload to the VIP-UHD-CTRL.

NOTE: The update takes approximately 2 minutes to complete, and the page refreshes to the Drag & Drop page once complete.