RTI VIP-UHD-TX/RX Firmware Version 2.3

LIST OF CHANGES IN THIS RELEASE (RELEASE v2.3)

-------------------------------------------------------

1. Ability to change polling frequency for large systems (75+ TX or RX) to improve stability.

Note: VIP-UHD-CTRL MUST be on version 2.03.

INSTALLATION REQUIREMENTS

-------------------------

Updating the VIP-UHD-CTRL requires:

1. The VIP-UHD-CTRL to be connected to the same Ethernet network (LAN) as the PC running APEX.

2. Requires the VIP-UHD-TX and VIP-UHD-RX to be connected to the VIP-UHD-CTRL via a switch on the video LAN.

NOTE: Hardwire connection is recommended.

FIRMWARE UPGRADE PROCEDURE

--------------------------

1. Download the VIP-UHD-TX and VIP-UHD-RX firmware file (.bin) from the RTI website to your computer.

2. Open APEX file with VIP-UHD-CTRL that needs to be updated.

3. Click on the driver dropdown and then click on the “RTI VIP-UHD-CTRL” driver

4. In the APEX workspace, click on “Launch Configuration” to launch the VIP-UHD-CTRL GUI.

5. Log into VIP-UHD-CTRL with username and password.

6. In the top bar, click “Update Firmware”.

7. In the top bar of the Update Firmware page, click on “Upload TX or RX Firmware”.

8. Select either the TX or RX .bin file downloaded to your computer for the unit.

9. Click on each “Update” button for the either the TX or RX depending on the file uploaded in the previous step.

10. Repeat steps 6 – 8 for either the TX or RX that wasn’t done the first time.

NOTE: If the TX file has been uploaded, only update the TX products. If the RX file has been uploaded, only update the RX products. Only one product can be updated at a time.